ISMS Class: INTERNAL

| Project Name/Code | AN14504/GALVmed/CTPM2 |
| --- | --- |
| Total No of stages | Main chain: 04, Side chain: 06 |
| Optimization report of the stage | CTPM2 |
| Starting date | 22.01.2021 |
| Completion date | 06.10.2024 |
| Laboratory Number | 210 |

Table of contents

Abbreviation table	6

1.0 Process optimization report of CTPM2	7

1.1 Process from the familiarization study of CTPM1	7

1.2 Synthetic scheme for CTPM1	7

1.3 Procedure	9

1.4 Conclusion:	12

1.5 The process from the familiarization study of CTPM2	13

1.6 Synthetic scheme for CTPM2	13

1.7 Procedure	14

1.8 Conclusion:	15

2.0 Optimization study of CTPM1 and CTPM2	16

2.1 Process optimization plan	16

2.1.1 Optimization of stoichiometry of Boc-L-valine for the CTPM1 reaction	16

2.1.2 Optimization of the stoichiometry of DCC for the CTPM2 reaction	19

2.1.3 Optimization of the stoichiometry of DMAP for the CTPM1 reaction	21

2.1.4 Optimization of the stoichiometry of 2-Propanol (IPA) for the CTPM2 reaction	24

2.1.5 Optimization of the stoichiometry of acetyl chloride for the CTPM2 reaction	26

2.2 What-if experiments	28

2.2.1 Stability of the CTPM1 reaction mass for an extended time at 25-35°C	28

2.2.2 Stability of CTPM1 organic layer at 50-55°C for 65 hours	28

2.2.3 Stability of CTPM1 viscous mass at 50-55°C for 65 hours	29

2.2.4 Effect of the CTPM1 reaction when THF contains 0.68% water content	30

2.2.5 Stability of CTPM2 reaction mass during extended maintenance at 25-30 °C	31

2.2.6 Stability of CTPM2 reaction mass during extended maintenance at 25-30°C	32

2.2.7 Impact of low purity of CTPM1 on CTPM2 intermediate purity	33

2.2.8 Impact of unreacted (6-(trifluoromethyl)pyridin-3-yl)methanol in CTPM1 on CTPM2 intermediate quality	34

2.2.9 Stability of CTPM2 intermediate during extended drying at 55-65 °C:	34

2.2.10 Stability of CTPM2 intermediate during extended drying at 55-65°C	35

2.3 Impurity profile of CTPM2	36

2.4 Lab trend: Impurity profile	39

2.5 Reprocess/rework procedure: Not applicable.	40

2.6 Recovery procedure: Not applicable	40

2.7 Solvent recovery: Not applicable	40

2.8 Equipment cleaning agent and process	40

2.9 Holding study	40

2.10 Hygroscopic study: Reference- CTPM2-001-0543658	41

2.11 Polymorphic study for CTPM2 (wherever applicable): Not applicable.	41

3.0 Final process description	41

3.1 Procedure	44

3.2 Note points	55

3.3 Lab batches experimental details	55

3.4 Analytical Data	56

3.5 Conclusion:	59

3.6 Summary of modifications in the CTPM2 process:	60

CHANGE HISTORY	61

## Abbreviation table

| Abbreviations | Full Description |
| --- | --- |
| HPLC | High-Performance Liquid Chromatography |
| THF | Tetrahydrofuran |
| Batch No. | Batch number |
| DCC | N, N′-dicyclohexylcarbodiimide |
| DCU | Dicyclohexylurea |
| DMAP | 4-Dimethylaminopyridine |
| GC | Gas chromatography |
| RPM | Rotation per minute |
| 1N HCl | 1Normal Hydrogen chloride |
| RRT | Relative retention time |
| W/W | Weight/weight |
| g | Gram |
| ND | Not detected |
| NA | Not applicable |
| °C | Degree Celsius |
| NMT | Not more than |
| IPC | In process control |
| SMUI | Single maximum unknown impurity |
| No. | Number |
| Eq. | Equivalent |
| IPA | 2-Propanol |
| DoE | Design of Experiments |
| RS by HPLC | Related substance by HPLC |

## 1.0 Process optimization report of CTPM2

The synthesis of CTPM2 involves two in situ steps, firstly the coupling of (tert-butoxycarbonyl)-L-valine with (6-(trifluoromethyl)pyridin-3-yl)methanol in the presence of N,N′-dicyclohexylcarbodiimide (DCC) and catalytic 4-dimethylaminopyridine (DMAP) to provide CTPM1 and then deprotection to remove N-Boc with in-situ generated hydrogen chloride using acetyl chloride and isopropanol (IPA) in isopropyl acetate solvent. A familiarization study of CTPM1 and CTPM2 was carried out separately using the procedure provided by GALVmed. As it has been challenging to obtain the CTPM1 intermediate as a solid during optimization studies, it was telescoped with the CTPM2 stage. Based on the observations from the CTPM2 demonstration batches 1 and 2, and from the outcome of DoE, and OFAT studies, the CTPM1 and CTPM2 process have been revised. Two new impurities, the Monofluoro and Difluoro analogues of CTPM,2 have also been included in the CTPM2 specifications. Two lab assurance batches of CTPM1 and CTPM2 stages were performed with the revised process, and the details are included in this revised optimization report.

## 1.1 Process from the familiarization study of CTPM1

## 1.2 Synthetic scheme for CTPM1

Table 1-Raw materials:

| S. No. | Raw Materials | Qty | Unit (g/ mL) | Mol. Wt. | M. Mole | Mole ratio | Volumes/ Times | Batch No./ Lot No. | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | (6-(Trifluoro-methyl) pyridin-3-yl)methanol | 10.0 | g | 177.13 | 56.5 | 1.0 | -- | 202120106 | Starting material-1 |
| 2 | Boc-L-valine | 13.5 | g | 217.27 | 62.1 | 1.1 | -- | B-964/1P/03 | Starting material-2 |
| 3 | 4-Dimethylaminopyridine (DMAP) | 1.38 | g | 122.17 | 11.29 | 0.2 | -- | Avra lab | Catalyst |
| 4 | N,N′-Dicyclohexylcarbodiimide (DCC) | 12.8 | g | 206.33 | 62.1 | 1.1 | -- | Avra lab | Reagent |
| 5 | Tetrahydrofuran lot-1 | 50.0 | mL | - | - | -- | 5.0 volumes | -- | Solvent for reaction |
| 6 | Tetrahydrofuran lot-2 | 20.0 | mL | - | - | -- | 2.0 volumes | -- | Solvent for reaction |
| 7 | Tetrahydrofuran lot-3 | 25.0 | mL | - | - | -- | 2.5 volumes | -- | Solvent for reaction |
| 8 | n-Heptane | 100.0 | mL | - | - | -- | 10.0 volumes | -- | Solvent for workup |
| 9 | Isopropyl acetate lot-1 | 50.0 | mL | - | - | -- | 5.0 volumes | -- | Solvent for workup |
| 10 | Isopropyl acetate lot-2 | 50.0 | mL | - | - | -- | 5.0 volumes | -- | Solvent for workup |
| 11 | Isopropyl acetate lot-3 | 50.0 | mL | - | - | -- | 5.0vol | -- | Solvent for workup |
| 12 | Isopropyl acetate lot-4 | 20.0 | mL | - | - | -- | 2.0 volumes | -- | Solvent for workup |
| 13 | 1M HCl solution lot-1 | 25.0 | mL | - | - | -- | 2.5 volumes | -- | For the organic layer wash |
| 14 | 1M HCl solution lot-2 | 25.0 | mL | - | - | -- | 2.5 Volumes | -- | For the organic layer wash |
| 15 | 8%Sodiumbicarbonate solution | 25.0 | mL | - | - | -- | 2.5 volumes | -- | For organic layer washing |
| 16 | Treated water | 25.0 | mL | - | - | -- | 2.5 volumes | -- | For workup |

## 1.3 Procedure

- Charged tetrahydrofuran lot-1 into the round bottom flask under a nitrogen atmosphere.

- Charged (tert-butoxycarbonyl)-L-valine.

- Stirred for 5-10 minutes.

- Charged 4-dimethylaminopyridine (DMAP).

- Stirred for 5-10 minutes.

- Charged (6-(trifluoromethyl)pyridin-3-yl)methanol.

- Stirred for 5-10 minutes.

- Cooled the reaction mass to -5 to 0°C.

- Prepared a solution of N,N′-dicyclohexylcarbodiimide (DCC), and tetrahydrofuran lot-2 under a nitrogen atmosphere.

- Charged the above-prepared solution dropwise into the reaction mass below 10°C under a nitrogen atmosphere.

(Note: Exothermic, the temperature increased from -1 to 2.2°C. White precipitation was observed)

- Stirred the reaction mass for 1-2 hours at -5 to 0°C.

- Temperature allowed to come to 25-35°C

- Stirred for 2-4 hours at 25-35°C.

- Checked TLC.

Note: Starting material was absent.

- Filtered the reaction mass.

Note: The product was in the filtrate

- Washed the solid with tetrahydrofuran, Lot-3.

- Suction-dried till no further filtrate or wash is collected.

- The filtrate was transferred back to the round bottom flask.

- Diluted with n-heptane.

- Stirred for 5-10 minutes.

- Charged 1N HCl lot-1 solution into reaction mass.

- Stirred for 5-10 minutes.

- Separated the upper organic layer and lower aqueous layer.

Note: The product was in the upper organic layer

- Charged the organic layer into the round bottom flask.

- Charged 1N HCl lot-2 solution into reaction mass.

- Stirred for 5-10 minutes.

- Separated the upper organic layer and lower aqueous layer.

Note: The product was in the upper organic layer.

- Charged the organic layer into the round bottom flask.

- Charged 8% sodium bicarbonate solution into the reaction mass.

- Stirred for 5-10 minutes.

- Separated the upper organic layer and the lower aqueous layer

Note: The product was in the upper organic layer.

- Charged the organic layer into the round bottom flask.

- Charged treated water into the above organic layer.

- Stirred for 5-10 minutes.

- Separated the upper organic layer and lower aqueous layer.

Note: The product was in the upper organic layer.

- Organic layer distilled under vacuum at below 45°C up to 4-5 volumes

- Charged isopropyl acetate lot-1.

- Distilled under vacuum at below 45°C up to 4-5 volumes.

- Charged isopropyl acetate lot-2.

- Distilled under vacuum at below 45°C up to 4-5 volumes.

- Charged isopropyl acetate lot-3.

- Distilled under vacuum at below 45°C up to 4-5 volumes.

- Cooled the mass to -5 to 5°C.

- Stirred for 1-2 hours (solid precipitation observed).

- Filtered the solid.

- Washed the filter cake using isopropyl acetate lot-4 (precooled).

- Unloaded the product.

Table 2

| Batch No. | Batch size (g) | Output (g) | Crude yield (%) | Purity (%) | Remarks |
| --- | --- | --- | --- | --- | --- |
| CTPM/ CTPM1-002 | 10.0 | 21.25 | 100.0 | 98.44 | Confirmed the product formation by 1H NMR, mass spectrometry, and HPLC. |

## 1.4 Conclusion:

Based on the above familiarization study, optimization studies for the CTPM1 can be considered to make it more robust and reproducible at the plant scale.

Recommendations based on the familiarization experiment

In-process checks for dicyclohexyl urea (DCU) content with appropriate analytical techniques can be incorporated into the process, as it is not detected in the existing analytical method.

## 1.5 The process from the familiarization study of CTPM2

## 1.6 Synthetic scheme for CTPM2

Table 3 CTPM2 Raw materials

| S. No. | Raw Materials | Qty | Unit (g/ mL) | Mol. Wt. | M. Mole | Mole ratio | Volumes/ Times | Batch No./ grade | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM1 (solution in isopropyl acetate) | 12.7 | g | 376.37 | 33.7 | 1.0 | - | CTPM1-002 | Starting material |
|  | Isopropyl alcohol (IPA) | 24.3 | g | 60.09 | 337.0 | 12.0 | - | LR grade | Reagent |
|  | Acetyl chloride (AcCl) | 26.5 | g | 74.49 | 405.0 | 10.0 | - | LR grade | Reagent |
|  | Isopropyl acetate Lot-1 | 196.8 | mL | - | - | - | 15.5 volumes | - | For reaction |
|  | Isopropyl acetate Lot-2 | 25.4 | mL | - | - | - | 2.0 volumes | - | For wet cake washing |
|  | Isopropyl acetate Lot-3 | 25.4 | mL | - | - | - | 2.0 volumes | - | For wet cake washing |
|  | Isopropyl acetate Lot-4 | 25.4 | mL | - | - | - | 2.0 volumes | - | For wet cake washing |

## 1.7 Procedure

- Charged isopropyl acetate lot-1 under a nitrogen atmosphere.

- Charged isopropyl alcohol at 25-35°C.

- Stirred reaction mass for 5-10 minutes.

- Cooled the reaction mass to -5 to 0°C.

- Charged acetyl chloride dropwise to the above solution at -5 to 10°C.

- Stirred for 1-2 hours at -5 to 10°C.

- Charged CTPM1 solution dropwise to the above solution at -5 to 10°C.

- Stirred for 15-30 minutes at -5 to 10°C.

- Temperature allowed to raise to 25-35°C

- Stirred reaction mass for 18-24 hours at 25-35°C

- Filtered the product under a nitrogen atmosphere.

Note: CTPM2 material is hygroscopic.

- Washed wet cake using isopropyl acetate lot-2, lot-3 and lot-4, under an inert atmosphere.

- Suction dried for 30-60 minutes.

- Unloaded the material under an inert atmosphere.

- Dried the solid at 45°C under a vacuum > 550 mm Hg.

- Send the sample for analysis.

Table 4

| Batch No. | Batch size (g) | Output (g) | Yield (%) | Purity (%) | Remarks |
| --- | --- | --- | --- | --- | --- |
| CTPM/CTPM2-001 | 12.7 | 9.8 | 93.33 | 99.80 | The product was confirmed by 1H-NMR, mass spectrometry, and purity analysis. |

## 1.8 Conclusion:

Based on the above familiarization study, the formation of the desired product (CTPM2) is confirmed by 1H NMR and mass spectrometry. The above process can be considered for further optimization to allow more robust and reproducible production at the plant scale.

## 2.0 Optimization study of CTPM1 and CTPM2

## 2.1 Process optimization plan

Based on the familiarization study, the following parameters were considered for optimizing CTPM1 and CTPM2.

2.1.1. Optimization of the stoichiometry of Boc-L-valine for the CTPM1 reaction

2.1.2. Optimization of the stoichiometry of DCC for the CTPM1 reaction

2.1.3. Optimization of the stoichiometry of DMAP for the CTPM1 reaction

2.1.4. Optimization of the stoichiometry of 2-Propanol (IPA) for the CTPM2 reaction

2.1.5. Optimization of the stoichiometry of acetyl chloride for the CTPM2 reaction

## 2.1.1 Optimization of stoichiometry of Boc-L-valine for the CTPM1 reaction

During familiarization studies, 1.1 mole equivalents of Boc-L-valine were used for the CTPM1 reaction. To check the effect of varied stoichiometry, reactions were performed with 1.10, 1.15, 1.20, 1.25, and 1.30 equivalents of Boc-L-valine. The standard CTPM1 reaction is performed using 1.20 equivalents of Boc-L-valine. All other raw materials were kept consistent with the finalized process. The experiment results are summarized in the below tables 5 and 6.

Table 5 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | Boc-L-valine Eq. | Purity by HPLC |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |  |
| CTPM/CTPM2-NB1-97 | 40.0 | 1.20 (standard) | 99.1 | 0.02 | 0.12 | 0.10 | 0.19 |  | Standard, reaction mass (IPC-02) |
|  |  |  | 99.5 | ND | 0.02 | 0.10 | 0.20 |  | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB1-07 | 5.0 | 1.10 | 97.9 | 0.04 | 0.10 | 0.86 | 0.14 |  | Reaction mass (IPC-02) |
|  |  |  | 98.6 | ND | ND | 0.43 | 0.18 |  | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-78 | 5.0 | 1.15 | 99.3 | 0.05 | ND | ND | ND |  | Reaction mass (IPC-02) |
|  |  |  | 99.2 | ND | ND | ND | ND |  | After the workup (CTPM1 organic layer IPC-06) |
| CTPM/CTPM2-NB2-79 | 5.0 | 1.25 | 99.1 | 0.01 | 0.04 | ND | ND |  | Reaction mass (IPC-02) |
|  |  |  | 99.5 | ND | ND | ND | ND |  | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB3-10 | 5.0 | 1.30 | 99.0 | 0.02 | 0.02 | ND |  | 0.23 | Reaction mass (IPC-02) |
|  |  |  | 98.6 | ND | ND | ND |  | 0.34 | After workupk (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  |  | NLT 93.0 | NMT 0.50 | NA |  |  |  |  |

Table 6 (CTPM2 impurity profile)

| Batch No. | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | CTPM1 | SMUI |  |  |
| CTPM/CTPM2-NB1-97 | 99.4 | ND | ND | ND | 0.28 | 0.02 | ND | ND | 0.05 0.18 RRT | ND | 84.4 |
| CTPM/CTPM2-NB1-07 | 99.2 | ND | ND | ND | 0.36 | 0.04 | ND | ND | 0.13 0.60 RRT | ND | 78.1 |
| CTPM/CTPM2-NB2-78 | 99.6 | 0.01 | ND | 0.02 | 0.23 | 0.02 | ND | ND | 0.07 1.28 RRT | ND | 84.1 |
| CTPM/CTPM2-NB2-79 | 99.6 | ND | ND | 0.02 | 0.17 | 0.03 | ND | ND | 0.07 1.28 RRT | ND | 84.4 |
| CTPM/CTPM2-NB3-10 | 99.6 | ND | ND | ND | 0.23 | 0.04 | ND | ND | 0.06 1.28 RRT | ND | 83.6 |
| Specification (%w/w) | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |

* Purity is not part of the specification

Conclusion:

Based on experimental data, using 1.10 equivalents led to a slight increase in impurity levels (0.77 RRT) in intermediate CTPM1. Although these impurities were removed during CTPM2 isolation, the final product meeting quality specifications showed a 5% reduction in overall yield. Additionally, a variation in the 0.93 RRT impurity was observed in the isolated CTPM2 solid. The formation of this impurity primarily results from cyclohexylamine, an impurity in DCC which reacts with Boc-L-valine. Therefore, a minimum of 1.2 equivalents of Boc-L-valine is required to consistently achieve the desired yield and quality as per predefined specifications.

## 2.1.2 Optimization of the stoichiometry of DCC for the CTPM2 reaction

During familiarization studies of the CTPM1 reaction, 1.1-mole equivalents of the DCC reagent were used for the reaction. To evaluate the impact of different DCC equivalents, reactions were performed with 1.10, 1.15, 1.20, 1.25, and 1.30 equivalents of DCC. The standard CTPM1 reaction is conducted using 1.20 equivalents of DMAP. All other raw materials remained consistent with the finalized process. The experiment results are summarized in the below tables 7 and 8.

Table 7 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | DCC Eq. | Purity by HPLC |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 |  | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM2-NB1-97 | 40.0 | 1.20 (standard) | 99.1 |  | 0.02 | 0.12 | 0.10 | 0.19 | Standard, reaction mass (IPC-02) |
|  |  |  | 99.5 |  | ND | 0.02 | 0.10 | 0.20 | After workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB1-57 | 50.0 | 1.10 | 99.2 |  | 0.06 | 0.05 | ND | 0.34 | Reaction mass (IPC-02) |
|  |  |  | 99.1 |  | 0.06 | ND | 0.04 | 0.02 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-84 | 5.0 | 1.15 | 99.1 |  | 0.01 | ND | ND | 0.17 | Reaction mass (IPC-02) |
|  |  |  | 99.5 |  | ND | ND | ND | 0.16 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-85 | 5.0 | 1.25 | 99.2 |  | 0.01 | ND | ND | 0.19 | Reaction mass (IPC-02) |
|  |  |  | 99.3 |  | ND | ND | ND | 0.19 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB3-11 | 5.0 | 1.30 | 98.5 |  | 0.11 | ND | ND | 0.37 | Reaction mass (IPC-02) |
|  |  |  | 99.3 |  | ND | ND | ND | 0.17 | After the workup (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  |  | NLT 93.0 | NMT 0.50 |  | NA |  |  |  |

Table 8(CTPM2 impurity profile)

| Batch No. | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | CTPM1 | SMUI |  |  |
| CTPM/CTPM2-NB1-97 | 99.4 | ND | ND | ND | 0.28 | 0.02 | ND | ND | 0.05 0.18 RRT | ND | 84.4 |
| CTPM/CTPM2-NB1-57 | 99.3 | 0.01 | ND | ND | 0.30 | 0.19 | ND | ND | 0.04 0.37 RRT | ND | 85.2 |
| CTPM/CTPM2-NB2-84 | 99.1 | ND | ND | 0.02 | 0.43 | 0.04 | ND | ND | 0.06 1.30 RRT | ND | 83.2 |
| CTPM/CTPM2-NB2-85 | 99.7 | ND | ND | 0.02 | 0.15 | 0.04 | ND | ND | 0.06 1.30 RRT | ND | 83.4 |
| CTPM/CTPM2-NB3-11 | 99.5 | ND | ND | ND | 0.20 | 0.05 | ND | ND | 0.06 1.27 RRT | ND | 83.9 |
| Specification (%w/w) | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |

* Purity is not part of the specification

Conclusion:

The formation of the 0.93 RRT impurity is primarily attributed to the presence of cyclohexylamine as an impurity in DCC and is not influenced by the amount of DCC used. Additionally, the use of 1.10 and 1.30 equivalents of DCC resulted in a slight increase in the 1.24 RRT impurity, with levels rising by 0.34% and 0.37%, respectively. This was accompanied by a 0.11% increase in compound-7 in CTPM1 and a 0.19% increase in CTPM2. Notably, the 1.24 RRT impurity was effectively purged during the isolation of CTPM2. Based on experimental data, all CTPM2 reactions consistently met predefined specifications for both reaction profile and yield. Therefore, the use of 1.2 mole equivalents of DCC is finalized for the CTPM1 reaction.

## 2.1.3 Optimization of the stoichiometry of DMAP for the CTPM1 reaction

During familiarization studies of the CTPM1 reaction, 0.20 mole equivalent (catalytic amount) of DMAP was used for the CTPM1 reaction. To evaluate the impact of different DMAP equivalents, reactions were performed with 0.10, 0.15, 0.20, 0.25, and 0.30 equivalents of DMAP. The standard CTPM1 reaction is conducted using 0.20 equivalents of DMAP. All other raw materials remained consistent with the finalized process. The experiment results are summarized in the below tables 9 and 10.

Table 9 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | DMAP Eq. | Purity by HPLC |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM2-NB1-97 | 40.0 | 0.20 (standard) | 99.1 | 0.02 | 0.12 | 0.10 | 0.19 | Standard, reaction mass (IPC-02) |
|  |  |  | 99.5 | ND | 0.02 | 0.10 | 0.20 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-82 | 5.0 | 0.10 | 99.2 | 0.01 | ND | ND | 0.23 | Reaction mass (IPC-02) |
|  |  |  | 97.9 | ND | ND | ND | 0.18 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-80 | 5.0 | 0.15 | 98.9 | 0.03 | ND | ND | 0.16 | Reaction mass (IPC-02) |
|  |  |  | 99.4 | ND | ND | ND | 0.16 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-81 | 5.0 | 0.25 | 99.2 | ND | ND | ND | 0.14 | Reaction mass (IPC-02) |
|  |  |  | 99.1 | ND | ND | ND | 0.14 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-83 | 5.0 | 0.30 | 99.2 | 0.01 | ND | ND | 0.20 | Reaction mass (IPC-02) |
|  |  |  | 99.4 | ND | ND | ND | 0.20 | After the workup (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  |  | NLT 93.0 | NMT 0.50 | NA |  |  |  |

Table 10 (CTPM2 impurity profile)

| Batch No. | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | CTPM1 | SMUI |  |  |
| CTPM/CTPM2-NB1-97 | 99.2 | ND | ND | ND | 0.28 | 0.02 | ND | ND | 0.05 0.18 RRT | 0.02 | 84.4 |
| CTPM/CTPM2-NB2-82 | 99.7 | ND | ND | 0.02 | 0.19 | 0.06 | ND | ND | 0.04 1.30 RRT | 0.01 | 83.2 |
| CTPM/CTPM2-NB2-80 | 99.4 | ND | ND | 0.02 | 0.26 | 0.09 | ND | ND | 0.04 1.30 RRT | ND | 82.6 |
| CTPM/CTPM2-NB2-81 | 99.8 | ND | ND | 0.02 | 0.04 | 0.04 | ND | ND | 0.06 1.30 RRT | ND | 82.9 |
| CTPM/CTPM2-NB2-83 | 99.6 | ND | ND | 0.02 | 0.25 | 0.06 | ND | ND | 0.06 1.30 RRT | ND | 83.0 |
| Specification (%w/w) | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |

* Purity is not part of the specification

Conclusion:

Experimental data consistently demonstrate that all CTPM2 reactions meet predefined specifications for impurity profile and yield when using 0.10 to 0.30 equivalents of DMAP. However, a variation in the 0.93RRT impurity has been observed. Notably, the formation of this impurity is independent of the DMAP quantity used. The 0.93RRT impurity primarily generates due to the presence of cyclohexylamine, an impurity in DCC, which reacts with Boc-L-valine to form this 0.93RRT impurity. Importantly, the 0.93RRT impurity observed in CTPM1 during DoE studies is unrelated to the one seen in CTPM2. Furthermore, this impurity is effectively purged during the isolation process in CTPM2. Therefore, the use of 0.2 mole equivalents of DMAP is finalized for the CTPM1 reaction.

## 2.1.4 Optimization of the stoichiometry of 2-Propanol (IPA) for the CTPM2 reaction

During the familiarization studies, 12.0-mole equivalents of isopropanol were used with respect to CTPM1 for the CTPM2 reaction. To check the effect of varying 2-Propanol (IPA) equivalents, reactions were performed with 10.0, 11.0, 12.0, 13.0, and 14.0 equivalents of 2-Propanol (IPA). All other raw materials were kept consistent with the finalized process. The experiment results are summarized in the below tables 11 and 12.

Table 11 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | Purity by HPLC |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM2-NB1-97 | 40.0 | 99.1 | 0.02 | 0.12 | 0.10 | 0.19 | Standard, reaction mass (IPC-02) |
|  |  | 99.5 | ND | 0.02 | 0.10 | 0.20 | After the workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-88 | 75.0 | 98.7 | 0.01 | ND | ND | 0.18 | Reaction mass (IPC-02) |
|  |  | 99.3 | ND | ND | ND | 0.18 | After the workup (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  | NLT 93.0 | NMT 0.50 | NA |  |  |  |

Note: The CTPM1 organic layer corresponding to CTPM2-NB2-88 was prepared as a stock solution and utilized to provide isopropanol (IPA) equivalents during the transformation of CTPM1 into CTPM2, as detailed in Table 12.

Table 12(CTPM2 impurity profile)

| Batch No. | IPA eq | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) | Remarks/ CTPM1 in reaction limit (NMT 1.0%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Comp.-7 | Boc-L-valine | CTPM1 | SMUI |  |  |  |
| CTPM/CTPM2-NB1-97 | 12.0 (standard) | 99.4 | ND | ND | ND | 0.28 | 0.02 | ND | ND | 0.05 0.18 RRT | ND | 84.4 | CTPM1:0.03% |
| CTPM/CTPM2-NB3-02 | 10.0 | 99.5 | ND | ND | 0.02 | 0.29 | 0.03 | ND | ND | 0.07 1.29 RRT | 0.04 | 84.6 | CTPM1: ND |
| CTPM/CTPM2-NB3-03 | 11.0 | 99.5 | ND | ND | 0.02 | 0.31 | 0.03 | ND | ND | 0.07 1.29 RRT | 0.05 | 85.2 | CTPM1: ND |
| CTPM/CTPM2-NB3-04 | 13.0 | 99.5 | ND | ND | 0.02 | 0.29 | 0.03 | ND | ND | 0.07 1.29 RRT | 0.02 | 85.7 | CTPM1: ND |
| CTPM/CTPM2-NB3-01 | 14.0 | 99.4 | ND | ND | 0.02 | 0.34 | 0.03 | ND | ND | 0.07 1.29 RRT | 0.03 | 83.1 | CTPM1: ND |
| Specification (%w/w) |  | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |  |

* Purity is not part of the specification

Conclusion:

Based on the above results, the CTPM2 reaction profile with 10.0, 11.0, 12.0, 13.0, and 14.0 equivalents of 2-Propanol (IPA) was comparable with the profile seen using the finalized equivalents (12.0 equivalents) of 2-Propanol (IPA).

## 2.1.5 Optimization of the stoichiometry of acetyl chloride for the CTPM2 reaction

During the familiarization studies, 10.0-mole equivalents of acetyl chloride were used with respect to CTPM1 for the CTPM2 reaction. To check the effect of varying acetyl chloride equivalents, 8.0, 9.0, 11.0, and 12.0 equivalents of acetyl chloride were used. All other raw materials remained consistent with the finalized process. The experiment results are summarized in the below tables 13 and 14.

Table 13 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | Purity by HPLC |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM2-NB1-97 | 40.0 | 99.1 | 0.02 | 0.12 | 0.10 | 0.19 | Standard, reaction mass (IPC-02) |
|  |  | 99.5 | ND | 0.02 | 0.10 | 0.20 | After workup (CTPM1 organic layer-IPC-06) |
| CTPM/CTPM2-NB2-88 | 75.0 | 98.7 | 0.01 | ND | ND | 0.18 | Reaction mass (IPC-02) |
|  |  | 99.3 | ND | ND | ND | 0.18 | After workup (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  | NLT 93.0 | NMT 0.50 | NA |  |  |  |

Note: The CTPM1 organic layer corresponding to CTPM2-NB2-88 was prepared as a stock solution and utilized to provide acetyl chloride equivalents during the transformation of CTPM1 into CTPM2, as detailed in Table 14.

Table 14 (CTPM2 impurity profile)

| Batch No. | Acetyl chloride eq | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) | Remarks/ CTPM1 in reaction limit (NMT 1.0%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Comp.-7 | Boc-L-valine | CTPM1 | SMUI |  |  |  |
| CTPM/CTPM2-NB1-97 | 10.0 (standard) | 99.4 | ND | ND | ND | 0.28 | 0.02 | ND | ND | 0.05 at 0.18 RRT | ND | 84.4 | 0.03% |
| CTPM/CTPM2-NB2-96 | 8.0 | 99.5 | ND | ND | 0.02 | 0.30 | 0.03 | ND | ND | 0.06 at 1.29 RRT | 0.02 | 83.7 | 2.93% |
| CTPM/CTPM2-NB2-97 | 9.0 | 99.5 | ND | ND | 0.02 | 0.29 | 0.04 | ND | ND | 0.06 at 1.29 RRT | 0.02 | 84.6 | 1.45% |
| CTPM/CTPM2-NB2-98 | 11.0 | 99.4 | ND | ND | 0.02 | 0.35 | 0.02 | ND | ND | 0.06 at 1.29 RRT | 0.02 | 85.5 | 0.07% |
| CTPM/CTPM2-NB2-99 | 12.0 | 99.4 | ND | ND | 0.02 | 0.32 | 0.04 | ND | ND | 0.06 at 1.29 RRT | 0.03 | 84.8 | 0.02% |
| Specification (%w/w) |  | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |  |

* Purity is not part of the specification

Conclusion:

Based on experimental data, the use of 8 and 9 equivalents of acetyl chloride resulted in CTPM1 content of 2.93% and 1.45%, respectively (Limit NMT: 1.0%). However, CTPM1 was effectively removed during the isolation of CTPM2, a final product that met quality specifications. This process led to a 1% reduction in overall yield. A total of four DoE (Design of Experiments) reactions were conducted using 8 equivalents of acetyl chloride CTPM2 reaction did not consistently meet specifications, especially CTPM1 content. Nevertheless, DoE results indicated that a minimum of 10 molar equivalents of acetyl chloride is required to reliably achieve both the desired yield and product quality in accordance with predefined specifications. (Ref: DOE report  RP-CDM-001506-1.0).

## 2.2 What-if experiments

## 2.2.1 Stability of the CTPM1 reaction mass for an extended time at 25-35°C

As per the finalized process, the CTPM1 reaction mass was stirred at 25-35°C for 6-8 hours. To further check the effect of reaction time on CTPM1 purity, the reaction mass was stirred at 25-35°C for an extended period of 72 hours. The experiment results are summarized in the below table 15.

Table 15 (CTPM1 impurity profile)

| Batch No | Batch size (g) | Time (Hours) | HPLC Profile (% area) |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM1-011 | 2.0 | 2 | 97.5 | ND | 0.23 | 0.07 | 0.03 | The CTPM1 reaction mass is stirred at 25-35°C for 72 hours. (IPC-02) |
|  |  | 24 | 96.8 | ND | 0.13 | ND | 0.02 |  |
|  |  | 72 | 97.5 | ND | 0.14 | ND | ND |  |
| IPC limit (% area) |  |  |  | NMT 0.50% |  |  |  |  |

Conclusion: Based on the above experimental data, no negative impact on the quality of CTPM1 was observed after extended maintenance of 72 hours at 25-35°C.

## 2.2.2 Stability of CTPM1 organic layer at 50-55°C for 65 hours

To establish the stability of the CTPM1 organic layer (THF and n-heptane) at 50-55°C during extended distillation, the obtained organic layer was stirred at 50-55°C for 65 hours. The experiment results are summarized in the below table 16.

Table 16 (CTPM1 impurity profile)

| Batch No | Batch size (g) | Time (Hours) | HPLC Profile (% area) |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM1-012 | 2.0 | Initial | 97.7 | 0.05 | ND | 0.18 | 0.19 | CTPM1 organic layer stirred for 65 hours at 50-55°C |
|  |  | 5 | 97.9 | 0.04 | ND | 0.16 | 0.14 |  |
|  |  | 28 | 97.7 | 0.09 | ND | 0.18 | 0.17 |  |
|  |  | 52 | 97.7 | 0.05 | ND | 0.18 | 0.15 |  |
|  |  | 65 | 98.0 | ND | ND | 0.17 | ND |  |

Conclusion:

Based on the above experimental data, it is concluded that the CTPM1 organic layer is stable (no significant change in HPLC purity, i.e. quality) for up to 65 hours at 50-55°C during extended distillation.

## 2.2.3 Stability of CTPM1 viscous mass at 50-55°C for 65 hours

To establish the stability of the CTPM1 organic layer (isopropyl acetate) at 50-55°C during extended distillation, the obtained organic layer was stirred at 50-55°C for 65 hours. The experiment results are summarized in the below table 17.

Table 17 (CTPM1 impurity profile)

| Batch No | Batch size (g) | Time (Hours) | HPLC Profile (% area) |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM1-013 | 2.0 | Initial | 97.6 | 0.27 | ND | 0.17 | 0.16 | CTPM1 organic layer stirred for 65 hours at 50-55°C |
|  |  | 5 | 97.7 | 0.20 | ND | 0.15 | 0.15 |  |
|  |  | 28 | 97.7 | 0.21 | ND | 0.17 | 0.14 |  |
|  |  | 52 | 97.8 | 0.07 | ND | 0.16 | 0.15 |  |
|  |  | 65 | 97.6 | ND | ND | 0.16 | ND |  |

Conclusion:

Based on the above experimental data, it is concluded that the CTPM1 organic layer (isopropyl acetate) is stable (no significant change in HPLC purity, i.e. quality) for up to 65 hours at 50-55°C during extended distillation.

## 2.2.4 Effect of the CTPM1 reaction when THF contains 0.68% water content

As per the finalized process, the water content limit in THF at 0.2%. To check the effect of higher water content in the THF solvent, the experimental results are summarized in the below tables 18 and 19.

Table 18 (CTPM1 impurity profile)

| Batch No. | Input Compound-7 (g) | THF water content (%) | Purity by HPLC |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM1 | Compound-7 |  | 0.68 RRT | 0.77 RRT | 1.24 RRT |  |
| CTPM/CTPM2-NB2-91 | 5.0 | 0.68 | 98.9 | 0.02 |  | ND | ND | 0.18 | Reaction mass (IPC-02) |
|  |  |  | 99.3 | ND |  | ND | ND | 0.18 | After workup (CTPM1 organic layer-IPC-06) |
| IPC limit (% area) |  |  | NLT 93.0 |  | NMT 0.50 | NA |  |  |  |

Table 19 (CTPM2 impurity profile)

| Batch No. | RS by HPLC |  |  |  |  |  |  |  |  | DCU | Yield (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | CTPM1 | SMUI |  |  |
| CTPM/CTPM2-NB2-91 | 99.4 | ND | ND | 0.02 | 0.37 | 0.03 | ND | ND | 0.07 1.29 RRT | 0.03 | 83.7 |
| Specification (%w/w) | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 | NA |

* Purity is not part of the specification

Conclusion:

No abnormalities were observed with moisture content up to 0.68% in the THF solvent, and all specified specifications were met at this level. However, it is advised to keep the water content at 0.2% in the THF solvent (IPC-01).

## 2.2.5 Stability of CTPM2 reaction mass during extended maintenance at 25-30 °C

To check the stability of the CTPM2 reaction mass during prolonged maintenance, the reaction mass was stirred for 72hr at 25-30 °C. The experiment results are summarized in the below table 20.

Table 20- Old experiment

| Batch No. | Input (g) | Time (hr) | CTPM2 Purity (%) | Remarks |
| --- | --- | --- | --- | --- |
| CTPM/CTPM2-006 | 1.0 | 0 | 87.5 | The CTPM2 reaction mass is stirred for 72 hr. at 25-30 °C. |
|  |  | 6 | 88.6 |  |
|  |  | 24 | 93.3 |  |
|  |  | 72 | 96.7 |  |
|  |  | Isolated | 99.7 |  |

Conclusion: Based on the above experimental data, the CTPM2 reaction mass is stable up to 72hr at 25-30 °C.

## 2.2.6 Stability of CTPM2 reaction mass during extended maintenance at 25-30°C

As per the finalized process, the CTPM2 reaction mass was stirred at 25-35°C for 18-24 hours. To further check the effect of reaction time on CTPM2 purity, the reaction mass was stirred at 25-35°C for an extended period of 72 hours. The experimental results are tabulated in the table 21.

Table 21-New experiment

| Batch No | Batch size (g) | Time (Hours) | HPLC Profile (% area) |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTPM2 | CTPM1 | 1.14RRT | 1.38 RRT | 1.50 RRT |  |
| CTPM/CTPM2-NB3-31 | 2.0 | 24.0 | 99.3 | ND | 0.24 | 0.01 | 0.02 | CTPM2 reaction mass is stable for up to 72 hours at 25-35°C. |
|  |  | 72.0 | 99.2 | ND | 0.10 | ND | ND |  |

Conclusion: Based on the above experimental data, the CTPM2 reaction mass is stable for up to 72 hours at 25-35°C.

## 2.2.7 Impact of low purity of CTPM1 on CTPM2 intermediate purity

To check the impact of low purity of CTPM1 on the CTPM2 intermediate purity, experiments were conducted, and the results are tabulated below table 22.

Table 22

| Batch No. | Input (g) | CTPM1 (CTPM1-014) |  | CTPM2 | Yield (%) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Purity (%) | 0.94 RRT (%) | Purity (%) |  |  |
| CTPM/CTPM2-007 | 1.0 | 93.76 | 1.78 | 99.9 | 73.41 | Obtained desired purity at the CTPM2 stage using a less pure CTPM1 intermediate |

Conclusion: From the above experimental data, there is no impact of low purity of CTPM1 (at the purity level studied) on CTPM2 intermediate quality.

## 2.2.8 Impact of unreacted (6-(trifluoromethyl)pyridin-3-yl)methanol in CTPM1 on CTPM2 intermediate quality

To check the impact of unreacted (6-(trifluoromethyl)pyridin-3-yl)methanol on CTPM2 quality, the experiment was carried out with a higher level of (6-(trifluoromethyl)pyridin-3-yl)methanol in the CTPM1 intermediate. The experiment results are summarized in the below table 23.

Table 23

| Batch No. | Input (g) | CTPM1 (CTPM1-015) |  | CTPM2 |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Purity (%) | (6-(trifluoromethyl) pyridin-3 yl)methanol (%) | Purity (%) | ~0.97 RRT (%) |  |
| CTPM/ CTPM2-008 | 2.0 | 96.30 | 0.76 (0.53RRT) | 95.4 | 0.53 | Reaction mass purity profile |
|  |  |  |  | 99.5 | 0.42 | Isolated solid purity profile |

Conclusion: From the above experimental data, unreacted (6-(trifluoromethyl)pyridin-3-yl) methanol was observed at the CTPM2 stage. Hence, levels of this material should be controlled in the CTPM1 stage to below 0.50%.

## 2.2.9 Stability of CTPM2 intermediate during extended drying at 55-65 °C:

To check the stability of CTPM2 intermediate during extended drying, it was dried under vacuum at 55-65 °C for 72 hours and checked for purity after intervals. The experiment results are summarized in the below table 24.

Table 24-Old experiment

| Batch No | Batch size (g) | HPLC Profile (% area) |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | Time (hr) | CTPM2 purity (%) |  |
| CTPM/CTPM2-016 | 2.0 | 0 | 99.9 | CTPM2 extended drying stability at 55-65 °C |
|  |  | 7 | 99.9 |  |
|  |  | 24 | 99.9 |  |
|  |  | 48 | 99.9 |  |
|  |  | 72 | 99.9 |  |

Conclusion: Based on the above experimental data, it is concluded that the CTPM2 intermediate is stable up to 72hr at 55-65 °C during extended drying.

## 2.2.10 Stability of CTPM2 intermediate during extended drying at 55-65°C

CTPM2 output material was subjected to prolonged drying for up to 72 hours at 55-65°C, and samples were withdrawn at regular intervals for analysis. The experimental results are tabulated below Table 25.

Table 25- New experiment

| Batch No. | Time (Hours) | RS by HPLC (% w/w) |  |  |  |  |  |  |  |  | DCU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTPM2 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | CTPM1 | SMUI |  |
| CTPM/CTPM2-NB3-32 | 24.0 | 99.8 | ND | ND | 0.02 | 0.09 | 0.03 | ND | ND | 0.02 | ND |
|  | 96.0 | 99.7 | ND | ND | 0.02 | 0.16 | 0.03 | ND | ND | 0.02 | ND |
| Specification (%w/w) |  | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.5 |

Conclusion: Based on the above experimental data, it is concluded that the CTPM2 intermediate is stable up to 72 hours at 55-65°C during extended drying.

## 2.3 Impurity profile of CTPM2

The following are the known impurities observed in the CTPM2 stage.

Table 26

| S. No. | IUPAC name / Internal code | Structure | RRT | Source of impurity |
| --- | --- | --- | --- | --- |
| 1 | N, N-dimethylpyridin-4-amine (DMAP) |  | ~0.23 | The catalyst used in the CTPM1 reaction |
| 2 | (6-(Trifluoromethyl)pyridin-3-yl)methanol/ Compound-7 |  | ~1.17 | Unreacted starting material-1 |
| 3 | Boc-L-valine |  | ~1.88 | Unreacted starting material-2 |
| 4 | Dicyclohexyl urea/DCU |  | NA | Byproduct of the DCC reagent formed during the reaction. |
| 5 | Monofluoro CTPM1 impurity |  | 0.50 | Carried over impurity from Compound-7 |
| 6 | Difluoro CTPM2 impurity |  | 0.76 | Carried over impurity from Compound-7 |
| 7 | (6-(Trifluoromethyl) pyridin-3-yl)methyl (tert-butoxycarbonyl)-L-valinate/CTPM1 |  | ~2.56 | Unreacted intermediate in the CTPM2 reaction. |
| 8 | (S)-2-amino-N-cyclohexyl-3-methylbutanamide (0.93 RRT Impurity of CTPM2) |  | 0.93 | Carried over impurity from DCC |

Note: The Separate w/w method is used for the analysis of DCU, hence RRT is not applicable.

## 2.4 Lab trend: Impurity profile

Impurity profile of CTPM1 in isopropyl acetate

Table 27

| Batch No | CTPM1 purity (%) | Impurity RRT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.50 (Compound-7) | 0.62 | 0.68 | 0.77 | 1.02 | 1.04 | 1.07 | 1.23 | Remarks |
|  |  | (%) | (%) | (%) | (%) | (%) | (%) | (%) | (%) |  |
| CTPM/CTPM2-NB1-96 | 99.46 | ND | 0.02 | ND | 0.10 | 0.09 | 0.10 | 0.06 | 0.17 | Obtained purities as per the trend |
| CTPM/CTPM2-NB1-97 | 99.45 | ND | 0.02 | 0.02 | 0.10 | 0.06 | 0.05 | 0.05 | 0.21 |  |

Compound-7: (6-(trifluoromethyl)pyridin-3-yl)methanol.

Impurity profile of CTPM2 (isolated intermediate):

Table 28

| Batch No | RS by HPLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTPM2 | 0.18 | DMAP | Mono fluoro analogue of CTPM2 | Di fluoro analogue of CTPM2 | 0.93 RRT | Compound-7 | Boc-L-valine | DCU | CTPM1 |
| CTPM/CTPM2-NB1-96 | 99.22 | 0.04 | ND | ND | ND | 0.38 | 0.02 | ND | 0.02 | ND |
| CTPM/CTPM2-NB1-97 | 99.37 | 0.03 | ND | ND | ND | 0.28 | 0.02 | ND | ND | ND |
| Specification (%w/w) | NA* | NMT 0.4 | NMT 0.4 | NMT 0.4 | NMT 0.6 | NMT 0.6 | NMT 0.6 | NMT 0.4 | NMT 0.5 | NMT 0.5 |

## 2.5 Reprocess/rework procedure: Not applicable.

## 2.6 Recovery procedure: Not applicable

## 2.7 Solvent recovery: Not applicable

## 2.8 Equipment cleaning agent and process

CTPM1 is liquid, and it is used as such for the next step. It is soluble in methanol and water. The solubility of CTPM2 was checked in solvents like methanol, water, dichloromethane, and isopropyl acetate. CTPM2 is soluble in 8 volumes of water at room temperature; hence, the following cleaning process will be applied for the CTPM2 stage (Ref: CTPM/CTPM2-NB1-01 & CTPM/CTPM2-NB1-43).

- Charge water into the reactor at 30±5ºC.

- Raise the temperature to reflux.

- Reflux for 1-2 hours.

- Cool to 30±5°C.

- Unload the water and discard it.

- Rinse the reactor with water.

- Unload water and send the sample to check the CTPM2 content by HPLC.

(Limit: CTPM2 content by UV method, NMT 9 ppm. If IPC fails, repeat the operation number 1 to 7 till IPC complies).

## 2.9 Holding study

  - CTPM1-RM holding study: AR No. 121080348

  - CTPM1-Intermediate holding study: AR No. 121080305 (organic layer)

  - CTPM2-RM holding study: AR No. 121080421

  - CTPM2-wet cake holding study: AR No. 121082195

  - CTPM2-Intermediate holding study: AR No. 121080767

## 2.10 Hygroscopic study: Reference- CTPM2-001-0543658

## 2.11 Polymorphic study for CTPM2 (wherever applicable): Not applicable.

## 3.0 Final process description

Reaction Scheme

Table 29- Raw materials

| S. No. | Raw Materials | Qty | Unit (g/ mL) | Mol. Wt. | M. Mole | Mole ratio | Vol/ Times | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | (6-(Trifluoromethyl) pyridin-3-yl)methanol | 40.0 | g | 177.13 | 226.0 | 1.0 | - | Key starting material-1 |
| 2 | Boc-L-valine Lot-1 | 58.9 | g | 217.27 | 271.0 | 1.2 | - | Key starting material -2 |
| 3 | Boc-L-valine Lot-2 | 4.91 | g | 217.27 | 22.58 | 0.1 | - | Key starting material -2 |
| 4 | Boc-L-valine Lot-3 | 4.91 | g | 217.27 | 22.58 | 0.1 | - | Key starting material -2 |
| 5 | 4-Dimethylaminopyridine (DMAP) | 5.52 | g | 122.17 | 45.2 | 0.2 | - | Catalyst |
| 6 | N,N′-Dicyclohexylcarbodiimide (DCC) lot-1 | 55.9 | g | 206.33 | 271.0 | 1.2 | - | Reagent |
| 7 | N,N′-Dicyclohexylcarbodiimide (DCC) lot-2 | 9.32 | g | 206.33 | 45.2 | 0.2 | - | Reagent |
| 8 | N,N′-Dicyclohexylcarbodiimide (DCC) lot-3 | 9.32 | g | 206.33 | 45.2 | 0.2 | - | Reagent |
| 9 | Isopropyl alcohol (IPA) | 163.0 | g | 60.09 | 2710.0 | 12.0 | - | For reaction |
| 10 | Acetyl chloride (AcCl) | 177.0 | g | 78.49 | 2258.0 | 10.0 | - | Reagent for the reaction |
| 11 | Conc hydrochloric acid lot-1 (HCl) | 10.0 | mL | - | - | - | 0.25 volumes | For organic layer washing |
| 12 | Conc hydrochloric acid lot-2 (HCl) | 10.0 | mL | - | - | - | 0.25 volumes | For organic layer washing |
| 13 | Sodium bicarbonate | 16.0 | g | - | - | - | 0.4 w/w | For organic layer washing |
| 14 | Tetrahydrofuran lot-1* | 80.0 | mL | - | - | - | 2.0 volumes | For cleaning |
| 15 | Tetrahydrofuran lot-2* | 20.0 | mL | - | - | - | 0.5 volumes | For cleaning |
| 16 | Tetrahydrofuran lot-3 | 200.0 | mL | - | - | - | 5.0 volumes | For reaction |
| 17 | Tetrahydrofuran lot-4 | 80.0 | mL | - | - | - | 2.0 volumes | For reaction |
| 18 | Tetrahydrofuran lot-5 | 20.0 | mL | - | - | - | 0.5 volumes | For reaction |
| 19 | Tetrahydrofuran lot-6 | 20.0 | mL | - | - | - | 0.5 volumes | For reaction |
| 20 | Tetrahydrofuran lot-7 | 80.0 | mL | - | - | - | 2.0 volumes | For washing |
| 21 | Tetrahydrofuran lot-8 | 200.0 | mL | - | - | - | 5.0 volumes | For slurry |
| 22 | Tetrahydrofuran lot-9 | 80.0 | mL | - | - | - | 2.0 volumes | For washing |
| 23 | n-Heptane | 400.0 | mL | - | - | - | 10.0 volumes | For workup |
| 24 | Isopropyl acetate lot-1 | 200.0 | mL | - | - | - | 5.0 volumes | Co-distillation |
| 25 | Isopropyl acetate lot-2 | 200.0 | mL | - | - | - | 5.0 volumes | Co-distillation |
| 26 | Isopropyl acetate lot-3 | 200.0 | mL | - | - | - | 5.0 volumes | Co-distillation |
| 27 | Isopropyl acetate lot-4 | 200.0 | mL | - | - | - | 5.0 volumes | Co-distillation |
| 28 | Isopropyl acetate lot-5 | 200.0 | mL | - | - | - | 5.0 volumes | Co-distillation |
| 29 | Isopropyl acetate lot-6 | 80.0 | mL | - | - | - | 2.0 volumes | Washing |
| 30 | Isopropyl acetate lot-7 | 80.0 | mL | - | - | - | 2.0 volumes | Washing |
| 31 | Treated water lot-1 | 100.0 | mL | - | - | - | 2.5 volumes | For 1N HCl Solution |
| 32 | Treated water lot-2 | 100.0 | mL | - | - | - | 2.5 volumes | For 1N HCl solution |
| 33 | Treated water lot-3 | 200.0 | mL | - | - | - | 5.0 volumes | For NaHCO3 solution |
| 34 | Treated water lot-4 | 100.0 | mL | - | - | - | 2.5 volumes | Work up |
| 35 | Isopropyl acetate Lot-8* | 80.0 | mL | - | - | - | 2.0 volumes | For cleaning |
| 36 | Isopropyl acetate Lot-9* | 20.0 | mL | - | - | - | 0.5 volumes | For cleaning |
| 37 | Isopropyl acetate Lot-10 | 1280.0 | mL | - | - | - | 32.0 volumes | For reaction |
| 38 | Isopropyl acetate Lot-11 | 160.0 | mL | - | - | - | 4.0 volumes | For wet cake washing |
| 39 | Isopropyl acetate Lot-12 | 160.0 | mL | - | - | - | 4.0 volumes | For wet cake washing |
| 40 | Isopropyl acetate Lot-13 | 160.0 | mL | - | - | - | 4.0 volumes | For wet cake washing |
| 41 | Isopropyl acetate Lot-14 | 160.0 | mL | - | - | - | 4.0 volumes | For slurry washing |
| 42 | Isopropyl acetate Lot-15 | 80.0 | mL | - | - | - | 2.0 volumes | For wet cake washing |

*Variable quantities based on equipment.

## 3.1 Procedure

| Operation number | Operation description |
| --- | --- |

- Charge tetrahydrofuran lot-1 into a round bottom flask.

- Raise the temperature to reflux (65±5°C).

- Maintain reflux for 15-30 minutes.

- Stop heating and cool the mass to 25-35°C.

- Unload the solvent and discard it.

- Rinse the round bottom flask with tetrahydrofuran lot-2 and unload the rinsed solvent.

- Send the rinsed solvent for water content as CTPM2-01 (Limit: NMT 0.20%) and methanol content as CTPM2-01a (Limit: NMT 100 ppm).

Note: If it complies, go to the next step. If IPC fails, repeat Op No 1 to 7 using a fresh lot of tetrahydrofuran, lot-1 and lot-2 till IPC complies.

Note: Reaction is moisture sensitive. If moisture levels are elevated, an additional lot of N,N′-dicyclohexylcarbodiimide (DCC) lot-2 may be required, and increased levels of an impurity at 0.94 RRT will be observed.

Note: Methanol contamination will generate L-valine methyl ester impurity.

- Charge (tert-butoxycarbonyl)-L-valine lot-1 under a nitrogen atmosphere.

- Charge tetrahydrofuran lot-3 into RBF under a nitrogen atmosphere.

- Stir reaction mass at 25-35°C for 5-10 minutes under a nitrogen atmosphere.

- Charge DMAP into the reaction mass at 25-35°C under a nitrogen atmosphere.

- Stir reaction mass at 25-35°C for 5-10 minutes under a nitrogen atmosphere.

- Charge (6-(trifluoromethyl)pyridin-3-yl)methanol at 25-35°C for 5-10 minutes under a nitrogen atmosphere.

- Stir reaction mass at 25-35°C for 5-10 minutes under a nitrogen atmosphere.

- Cool the reaction mass to -5 to 10°C.

- Prepare a solution of N,N′-dicyclohexylcarbodiimide (DCC) lot-1 in tetrahydrofuran lot-4 under a nitrogen atmosphere.

Note i) Warm DCC to 25-35°C and unload the liquid DCC reagent (DCC is a low melting solid).

ii) Rinse with tetrahydrofuran lot-4.

- Add the above-prepared solution slowly into the reaction mass at -5 to 10°C.

Note i) Addition is exothermic.

ii) Precipitation of dicyclohexyl urea (DCU) by-product is typically observed at this point.

- Stir the reaction mass for 1-2 hours at -5 to 10°C under a nitrogen atmosphere.

- Raise reaction mass temperature to 25-35°C.

- Stir reaction mass for 6-8 hours at 25-35°C under a nitrogen atmosphere.

Note: Preferably 25-30°C.

Note: Reaction mass stable for 72 hours.

- Send reaction mass sample for HPLC analysis as CTPM2-02 (Limit: (6-(trifluoromethyl)pyridin-3-yl)methanol content, NMT 0.50%).

Note: If it complies, go to op. No. 30. If it does not comply, go to op. No. 22.

Note 2: Reaction mass is heterogeneous. Collect the sample under stirring.

- Cool the reaction mass to -5 to 10°C.

- Charge (tert-butoxycarbonyl)-L-valine lot-2 under a nitrogen atmosphere.

- Prepare a solution of N,N′-dicyclohexylcarbodiimide (DCC) lot-2 in tetrahydrofuran lot-5 under a nitrogen atmosphere.

Note i) Warm N,N′-dicyclohexylcarbodiimide (DCC) to 25-35°C and unload the liquid DCC reagent (DCC is a low melting solid).

ii) Rinse with tetrahydrofuran lot-5.

- Add the above-prepared solution slowly into the reaction mass at -5 to 10°C.

Note i) Addition is exothermic.

- Stir the reaction mass for 45±15minutes at -5 to 10°C under a nitrogen atmosphere.

- Raise reaction mass temperature to 25-35°C.

Note: Preferably 25-30°C.

- Stir reaction mass for 1-2 hours at 25-35°C under a nitrogen atmosphere.

Note: Reaction mass stable for 72 hours.

- Send reaction mass sample for HPLC analysis as CTPM2-02. (Limit: (6-(trifluoromethyl)pyridin-3-yl)methanol content, NMT 0.50%).

Note-1: If it complies, go to op. No. 30. If it does not comply, repeat the op. no 22 to 29 using (tert-butoxycarbonyl)-L-valine lot-3, N,N′-dicyclohexylcarbodiimide (DCC) lot-3, and tetrahydrofuran lot-6 under nitrogen atmosphere till IPC complies.

Note 2: The reaction mass is heterogeneous in nature. Send a sample under stirring.

- Filter the reaction mass.

Note i) Product is in the filtrate.

ii) Solid is a byproduct of dicyclohexyl urea (DCU).

- Wash the filter cake with tetrahydrofuran lot-7.

- Suction dry till no further filtrate or wash is collected.

Note i) Product is in the filtrate, stored in a clean and dry container.

ii) Solid is a byproduct of dicyclohexyl urea (DCU).

- Charge wet DCU solid into the round bottom flask.

- Charge tetrahydrofuran lot-8 into RBF.

- Stir reaction mass at 25-35°C for 15-30 minutes.

- Filter the reaction mass.

Note i) Product is in the filtrate, stored in a clean and dry container.

ii) Solid is a by-product of dicyclohexyl urea (DCU).

- Wash the filter cake with tetrahydrofuran lot-9.

- Suction dry till no further filtrate or wash is collected.

- Charge filtrate op. no. 32 and 38 into the clean round bottom flask.

- Charge n-heptane into the round bottom flask.

- Stir for 5-10 minutes at 25-35°C.

- Prepare 1N hydrochloric acid (HCl) solution using Conc. HCl lot-1 and treated water lot-1.

- Charge the above prepared 1N HCl solution to the above round bottom flask.

- Stir for 10-20 minutes at 25-35°C.

- Settle reaction mass for NLT 30 minutes.

- Separate the upper organic layer and lower aqueous layer.

Note: The product is in the upper organic layer.

- Charge the organic layer into the round bottom flask.

- Prepare 1N HCl solution using Conc. HCl lot-2 and treated water lot-2.

- Charge the above prepared 1N HCl solution to the above round bottom flask.

- Stir for 10-20 minutes at 25-35°C.

- Settle reaction mass for NLT 30 minutes.

- Separate the upper organic and lower aqueous layers.

Note: The product is in the upper organic layer.

- Charge the organic layer into the round bottom flask.

- Prepare sodium bicarbonate solution using sodium bicarbonate and treated water lot-3.

- Charge prepared aqueous sodium bicarbonate solution into the organic layer.

- Stir for 10-20 minutes at 25-35°C.

- Settle reaction mass for not less than (NLT) 30 minutes.

- Separate the upper organic and lower aqueous layers

Note: The product is in the upper organic layer.

- Charge the organic layer into the round bottom flask.

- Charge treated water lot 4

- Stir for 10-20 minutes at 25-35°C.

- Separate the layers.

Note: The product is in the upper organic layer.

- Charge the organic layer into the round bottom flask.

- Distil out solvent to 4-5 volumes under a vacuum below 45°C.

Note: i) Organic layer stable at 50-55°C up to 65 hours.

ii) Add butylated hydroxytoluene (BHT) to the distillate receiver.

- Charge isopropyl acetate lot-1 into reaction mass below 45°C.

- Distil out solvent to 4-5 volumes under a vacuum below 45°C.

Note: Add butylated hydroxytoluene (BHT) to the distillate receiver for the entire distillation.

- Charge isopropyl acetate lot-2 into reaction mass below 45°C.

- Distil out solvent to 4-5 volumes under a vacuum below 45°C.

- Charge isopropyl acetate lot-3 into reaction mass below 45°C.

- Distil out solvent to 4-5 volumes under a vacuum below 45°C.

Note: Crude mass is stable at 50-55°C for 65 hours.

- Submit a sample for GC analysis to check THF content as CTPM2-03 (Limit: THF content by GC, NMT 1.0%).

Note: if it complies, go to the next operation. If it does not comply, charge isopropyl acetate lot-4 and distil solvent up to 4-5 volumes, and submit a sample for GC to check THF content. Repeat this process till IPC complies.

- Submit a sample for water content by KF as CTPM2-04 (Limit: NMT 0.20%)

Note: If it complies, go to the next operation. If it does not comply, charge isopropyl acetate lot-5 and distil solvent up to 4-5 volumes, and submit a sample for water content by KF as CTPM2-04. Repeat this operation till IPC complies.

- Cool the mass to -5 to 5°C.

- Stir for 1-2 hours at -5 to 5°C.

Note: Precipitation of a solid is observed

- Filter the obtained solid at -5 to 5°C.

- Wash the filter cake with precooled isopropyl acetate lot-6.

- Submit a sample for HPLC to check dicyclohexyl urea (DCU) content as CTPM2-05 (Limit: NMT 0.50%)

Note: If it complies, go to the next operation. If it does not comply, repeat operations 73 to 77 using isopropyl acetate lot-7 till IPC complies.

- Unload the CTPM1 solution into a clean container and record the weight as Xg.

- Submit a sample for purity by HPLC as sample no. CTPM2-06 (Limit: CTPM1 purity, NLT 93.0%).

- Submit a sample for HPLC by assay as CTPM2-IH-01 (Limit: Report the result).

CTPM2 reaction procedure:

- Charge isopropyl acetate lot-8 into the round bottom flask.

- Raise the temperature to reflux (85-90°C).

- Maintain reflux for 15-30 minutes.

- Stop heating and cool the mass to 25-35°C.

- Unload the solvent and discard it.

- Rinse the round bottom flask with isopropyl acetate lot-9 and unload the rinsed solvent.

- Send the rinsed solvent for water content as CTPM2-07 (Limit: NMT 0.20%).

Note: If it complies, go to the next step. If IPC fails, repeat Op No 81 to 87 using a fresh lot of isopropyl acetate lot-8 and lot-9 till IPC complies.

- Charge isopropyl acetate lot-10 under a nitrogen atmosphere.

- Charge isopropanol at 25-35°C under a nitrogen atmosphere.

- Stir for 5-10 minutes.

- Cool the reaction mass to -5 to 5°C.

- Add acetyl chloride dropwise to the above solution at -5 to 10°C.

- Stir for 30-60 minutes at -5 to 10°C.

- Charge CTPM1 solution obtained from the previous step (op. no. 78) dropwise to the above solution at -5 to 10°C.

Note: Use a clean addition funnel for the addition of CTPM1 solution. The addition funnel used for acetyl chloride should not be used as such.

- Stir for 15-30 minutes at -5 to 10°C.

- Raise reaction mass temperature to 25-35°C under nitrogen.

- Stir the reaction mass for 18-24 hours at 25-35°C

Note: Reaction mass stable at 25-35°C for 72hours.

- Send the sample for HPLC analysis as CTPM2-08 for CTPM1 content (Limit: NMT, 1.0%).

Note: if it complies, go to the next step. If IPC fails, continue stirring and send a sample every 4-6 hours till IPC complies.

- Filter the product under a nitrogen atmosphere.

Note: CTPM2 material is hygroscopic and should be handled under a nitrogen atmosphere.

- Wash the wet cake using isopropyl acetate lot-11, lot-12, and lot-13 under an inert atmosphere.

- Suction dry under a nitrogen atmosphere till no mother liquor is observed.

- Unload the material under an inert atmosphere.

Note: CTPM2 is hygroscopic; hence, wet cake should be handled under a nitrogen atmosphere to avoid degradation.

- Dry the solid under vacuum (NLT 600 mm Hg) for 16-20 hours at 60-65°C.

Note: 1. Dry the wet material under vacuum (NLT 600 mmHg) at 30±5°C for 4-6 hours.

2. Delump after every 4-6 hours under a nitrogen blanket (CTPM2 is hygroscopic in nature).

3. All operations should be done under an inert atmosphere since the product is hygroscopic. Release the vacuum under a nitrogen atmosphere.

4. CTPM2 is stable for 72 hours at 55-65°C during drying.

- Send the sample for related substance by HPLC analysis as CTPM2-09 *Purity: NLT 98.0 %, (Limit: Related substances by HPLC (% w/w), Method-I: i) DMAP: NMT 0.4 ii) Monofluoro analogue of CTPM2: NMT 0.4 iii) Difluoro analogue of CTPM2: NMT 0.4 iv) Compound 7: NMT 0.6 v) Boc-L-valine: NMT 0.4 vi) CTPM1: NMT 0.4 vii) 0.93 RRT impurity of CTPM2:NMT 0.6 viii) Individual unspecified impurities NMT 0.4; Method-II: i) Dicyclohexylurea: NMT 0.5

*Purity by Mass balance (100%-Total impurities)

Note: If it complies, go to op. No. 114. If it does not comply, go to op. No. 105.

- Charge CTPM2 solid into RBF under a nitrogen atmosphere.

- Charge isopropyl acetate lot-14 into RBF under a nitrogen atmosphere.

- Stir for 1-2 hours under a nitrogen atmosphere.

- Filter the product under a nitrogen atmosphere.

Note: CTPM2 material is hygroscopic and should be handled under a nitrogen atmosphere.

- Wash the wet cake using isopropyl acetate lot-15 under an inert atmosphere.

- Suction dry under a nitrogen atmosphere till no mother liquor is observed.

- Unload the material under an inert atmosphere.

Note: CTPM2 is hygroscopic; hence, wet cake should be handled under a nitrogen atmosphere to avoid degradation.

- Dry the solid under vacuum (NLT 600 mm Hg) for 16-20 hours at 60-65°C.

Note 1. Dry the wet material under vacuum (NLT 600 mmHg) at 30±5°C for 4-6 hours.

2. Delump after every 4-6 hours under a nitrogen blanket (CTPM2 is hygroscopic in nature)

3. All operations should be done under an inert atmosphere since the product is hygroscopic. Release the vacuum under a nitrogen atmosphere.

4. CTPM2 is stable for 72 hours at 55-65°C during drying.

- Send the sample for related substance by HPLC analysis as CTPM2-09 *Purity: NLT 98.0 %, (Limit: Related substances by HPLC (% w/w), Method-I: i) DMAP: NMT 0.4 ii) Monofluoro analogue of CTPM2: NMT 0.4 iii) Difluoro analogue of CTPM2: NMT 0.4 iv) Compound 7: NMT 0.6 v) Boc-L-valine: NMT: 0.4 vi) CTPM1: NMT 0.4 vii) 0.93 RRT impurity of CTPM2: NMT 0.6  viii) Individual unspecified impurities NMT 0.4; Method-II: i) Dicyclohexylurea: NMT 0.5

*Purity by Mass balance (100%-Total impurities)

Note: If it complies, go to op. No. 114. If it does not comply, repeat op. No. 105 to 113 using a fresh lot of isopropyl acetate, lot-14 and lot-15 till IPC complies.

- Send the sample for LOD by HMA as CTPM2-10 (Limit NMT 4.5%).

Note: If it complies, go to the next step. If it does not comply, continue drying under a vacuum and send a sample every 4-6 hours till IPC complies.

Note: All operations should be done under an inert atmosphere since the product is hygroscopic.

- Unload and pack the solid under a nitrogen atmosphere.

Note: Pack material in a white polyethene bag under a nitrogen atmosphere and tie it with a plastic strip. Keep it in a black polythene bag and tie it with a plastic strip. Keep the entire bag in a triple laminated cover and tie it. Keep this bag in an HDPE container along with silica packets, and put a clamp on the HDPE container.

- Send the sample for complete analysis.

Theoretical yield (g): Batch size (g) * 1.765

Yield range: 85±15%

## 3.2 Note points

- Op. no. 99 to 103, 108 to 112, and 115 should be carried out under a nitrogen atmosphere because CTPM2 material is hygroscopic and hydrolyses in the presence of water.

- CTPM2 material is hygroscopic and should be handled under a nitrogen atmosphere.

## 3.3 Lab batches experimental details

Table 30

| S. No. | Experiment No. | Input (g) | Output (g) | Yield (%) after purity correction | Purity by HPLC (%Area) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CTPM/CTPM2-NB1-96 | 40.0 | 60.3 | 85.41 | 99.0 | Obtained predefined yield and quality. |
| 2 | CTPM/CTPM2-NB1-97 | 40.0 | 59.6 | 84.42 | 99.3 |  |

## 3.4 Analytical Data

a) List of in-process samples and results:

Table 31

| IPC No | Analytical test. As per Spec. | Specification | CTPM/ CTPM2-NB1-96 | CTPM/ CTPM2-NB1-97 |
| --- | --- | --- | --- | --- |
| CTPM2-01 | THF water content by KF (% w/v) | NMT 0.20 | 0.104 | 0.104 |
| CTPM2-01a | Methanol content in THF by GC (ppm) | NMT 100 | ND | 0.004 |
| CTPM2-02 | (6-(Trifluoromethyl)pyridin-3-yl)methanol content by HPLC (% area) | NMT 0.50 | 0.011 | 0.017 |
| CTPM2-03 | THF content by GC (% w/w) | NMT 1.0 | 0.016 | 0.0041 |
| CTPM2-04 | Water content by KF (% w/v) | NMT 0.20 | 0.084 | 0.074 |
| CTPM2-05 | DCU content (% w/w) | NMT 0.50 | 0.036 | 0.048 |
| CTPM2-06 | Purity by HPLC (% area) | NLT 93.0 | 99.46 | 99.45 |
| CTPM2-IH-01 | Assay by HPLC (% w/w) | Report the Result | 34.37 | 34.96 |
| CTPM2-07 | Isopropyl acetate water content by KF (% w/v) | NMT 0.20 | 0.107 | 0.107 |
| CTPM2-08 | CTPM1 content by HPLC (% area) | NMT 1.0 | 0.027 | 0.026 |
| CTPM2-09 | *Purity by HPLC | NLT: 98.0 | 98.64 | 99.37 |
|  | Related substances by HPLC(% w/w) Method I: |  |  |  |
|  | - DMAP | NMT: 0.4 | 0.06 | Not detected |
|  | - Monofluoro analogue of CTPM2 | NMT: 0.4 | Not detected | Not detected |
|  | - Difluoro analogue of CTPM2 | NMT: 0.4 | Not detected | Not detected |
|  | - Compound-7 | NMT: 0.6 | 0.03 | 0.03 |
|  | - Boc-L-valine | NMT: 0.4 | Not detected | Not detected |
|  | - CTPM1 | NMT: 0.4 | Not detected | Not detected |
|  | - 0.93 RRT impurity of CTPM2 | NMT: 0.6 | 0.46 | 0.28 |
|  | - #Individual unspecified impurities | NMT: 0.4 | Complies | Complies |
|  | Method II : DCU Content by HPLC (% w/w) | NMT: 0.5 | 0.03 | ND |
| CTPM2-10 | LOD by HMA (% w/w) | NMT 4.5 | 3.3 | 3.4 |

b) Complete Analysis of intermediate:

Table 32

| S.No | Test | Specification | Experiment No. |  |
| --- | --- | --- | --- | --- |
|  |  |  | CTPM/ CTPM2-NB1-96 | CTPM/ CTPM2-NB1-97 |
| 1 | Appearance | White to off-white colour solid | White color solid | White color solid |
| 2 | Identification by HPLC | The retention time of the major peak in the chromatogram of the test solution shall match the retention time of the major peak in the chromatogram of the standard. | The retention time of the major peak in the chromatogram of the test solution matches the retention time of the major peak in the chromatogram of the system suitability solution. | The retention time of the major peak in the chromatogram of the test solution matches the retention time of the major peak in the chromatogram of the system suitability solution. |
| 3 | *Purity by HPLC | NLT: 98.0 | 99.22 | 99.37 |
| 4 | Related substances by HPLC (% w/w) Method I: |  |  |  |
|  | - DMAP | NMT: 0.4 | Not detected | Not detected |
|  | - Monofluoro analogue of CTPM2 | NMT: 0.4 | Not detected | Not detected |
|  | - Difluoro analogue of CTPM2 | NMT: 0.4 | Not detected | Not detected |
|  | - Compound-7 | NMT: 0.6 | 0.02 | 0.02 |
|  | - Boc-L-valine | NMT: 0.4 | Not detected | Not detected |
|  | - CTPM1 | NMT: 0.4 | Not detected | Not detected |
|  | - 0.93 RRT impurity of CTPM2 | NMT: 0.6 | 0.38 | 0.28 |
|  | - #Individual unspecified impurities | NMT: 0.4 | Complies | Complies |
| 5 | DCU content by HPLC (% w/w) | NMT: 0.5 | 0.03 | ND |
| 6 | Loss on drying by HMA (% w/w) | NMT: 5.0 | 3.3 | 3.4 |

*Purity by Mass balance (100%-Total impurities)

# Report the unspecified impurities that are more than or equal to the LOQ concentration, along with RRT

## 3.5 Conclusion:

Based on the above study, it is concluded that the CTPM2 process is optimized for the above scheme and can be used for further scale-up.

## 3.6 Summary of modifications in the CTPM2 process:

Table 33

| S.No | Process | Previous process (ALMAC) | Current process (APSL) |
| --- | --- | --- | --- |
| 1 | CTPM1 and CTPM2 | In the earlier Almac process, CTPM1 and CTPM2 are independent stages | The CTPM1 and CTPM2 stages are combined, CTPM1 is in-situ step |
| 2 | CTPM2 Raw material charging | Raw materials for CTPM2 are charged based on the CTPM1 assay by NMR | CTPM2 raw material are charged based on Compound-7 considering 100% assay for CTPM1 |
| 3 | The moisture content of tetrahydrofuran | Not part of the ALMAC tech pack. | Incorporated as an in-process check before batch start-up for tetrahydrofuran. Limit: NMT 0.20% |
| 4 | The methanol content by GC in THF after reactor cleaning. | Not part of the ALMAC process | Incorporated as an in-process check before batch start-up to avoid the L-valine methyl ester impurity formation (Limit: NMT 100 ppm) |
| 5 | THF content after distillation | THF content by the 1H NMR technique | THF content is checked by GC analysis, Limit NMT 1.0% (w/w) |
| 6 | Slurry wash to DCU solid | Not part of the ALMAC process | Slurry wash to the DCU solid is included in the process. |
| 7 | Dicyclohexyl urea (DCU) content | Not part of the ALMAC tech pack | Incorporated in-process control for DCU content by HPLC, Limit NMT 0.50% (w/w) |
| 8 | Assay by HPLC of CTPM1 | Assay by the 1H NMR technique | Assay by HPLC, Limit: Report the result. |
| 9 | The moisture content of isopropyl acetate is included as part of the in-process check. | Not part of the ALMAC tech pack. | Incorporated as IPC before batch start-up. Limit: NMT 0.20% |
| 10 | The drying temperature of CTPM2 | Limit, NMT: 1.0%, drying temperature 35-45°C | Limit, NMT: 5.0%, modified drying temp to 60-65°C. |
| 11 | Purity by HPLC of CTPM2 | DMAP, DCU, Monofluoro CTPM2, Difluoro CTPM2, and Boc-L-valine impurities were not detected in the HPLC method provided in the tech pack | A new HPLC method is developed, which can detect DMAP, Boc-L-valine, along with CTPM1, CTPM2, 6-(Trifluoromethyl)pyridin-3-yl)methanol, Monofluoro CTPM2, and Difluoro CTPM2. DCU quantification is performed in different analytical methods. |

## CHANGE HISTORY

Table 34

| Quality Record No | Brief details of changes |
| --- | --- |
| RP-CDM-000737-1.0 (CCF No.: CC1000049313) | New document (CCF No.: CC1000049313) |
| RP-CDM-000737-2.0 (CCF No.: CC1000049313) | CTPM2 optimization report (CCF No.: CC1000049313) - Corrected chemical name in the synthetic scheme in sections 1.1, 1.6, and 3.0. |
| RP-CDM-000737-3.0 (CCF No.: CC1000073751) | CTPM2 optimization report (CCF No.: CC1000073751) - Removed temperature from op. No. 8 and 81. - Operations no. 95 and 96 interchanged. - CTPM1 cleaning solvent is included in section 2.8. |
| RP-CDM-000737-4.0 (CCF No.: CC1000073751) | CTPM2 optimization report (CCF No.: CC1000073751) - In op. No. 72, the typo error is corrected as NLT 93.0% instead of NMT 93.0%. |
| RP-CDM-000737-5.0 (CCF No.: CC1000078716) | CTPM2 optimization report (CCF No.: CC1000078716) - In Section 3.1, op. No. 98: Limit of LOD by HMA in OPC no. CTPM2-10 revised to 4.5% from NMT 0.80%. - In section 3.5 a, Limit of LOD by HMA in IPC no. CTPM2-10 revised to NMT 4.5% from NMT 0.80%. - Section 3.5 b, Sr. no. 5: Limit of LOD revised to NMT 5.0% from NMT 1.0%. - Section 3.7, sr, no. 6, Limit of LOD is corrected to 5.0% in the current process (APSL) column. |
| RP-CDM-000737-6.0 (CCF No.: CC1000080579) (CCF No.: CC1000077504) (CCF No.: CC1000079658) | CTPM2 optimization report (CCF No.: CC1000080579) - In op. No. 94, suction dry time is removed. - In op. No. 97, the drying temperature is changed to 60-65°C. CTPM2 optimization report (CCF No.: CC1000077504) - In op. No. 96, updated revised limits of IPC no. CTPM2-09. - In section 3.5a, updated revised limits of IPC no. CTPM2-09 and the mentioned note below the table for IPC no. CTPM2-09. CTPM2 optimization report (CCF No.: CC1000079658) - In Section 2.8, the cleaning solvent changed to water, and the limit of CTPM2 content changed to NMT 9 ppm. - Reflux temperature removed from operation no. 2 |
| RP-CDM-000737-7.0 (CCF No.: CC1000097784) | CTPM2 optimization report (CCF No.: CC1000097784) - In section no. 3.0, in the raw material table, the input batch size is revised to 40.0g. Accordingly, all raw material quantities are revised. - The mole equivalent of the following raw materials is revised as follows, - (tert-Butoxycarbonyl)-L-valine (Boc-L-valine) lot-1: 1.2 eq. - (tert-Butoxycarbonyl)-L-valine (Boc-L-valine) lot-2: 0.1 eq. - (tert-Butoxycarbonyl)-L-valine (Boc-L-valine) lot-3: 0.1 eq. - N,N′-Dicyclohexylcarbodiimide (DCC) lot-1: 1.2 eq. - N,N′-Dicyclohexylcarbodiimide (DCC) lot-2: 0.2 eq. - N,N′-Dicyclohexylcarbodiimide (DCC) lot-3: 0.2 eq. - Sodium bicarbonate: 0.4 w/w - Tetrahydrofuran lot-6: 0.5 vol - Tetrahydrofuran lot-7: 2.0 vol - Tetrahydrofuran lot-8: 5.0 vol - Tetrahydrofuran lot-9: 2.0 vol. - Treated water lot-3: 5 vol. - Section 3.1, op. No. 7, new IPC-CTPM2-1a, is included in the process to check the methanol content in THF by GC (NMT 100 ppm). - Section 3.1, new lots of Boc-L-valine lot-2, lot-3 and DCC lot-3 are included, and accordingly, operations are updated. - In section 3.1, op. No. 104, IPC no. CTPM2-09 is submitted after drying wet; accordingly, operations are corrected in the process. - Section 3.1, op. No. 33 to 38, slurry wash to DCU solid using THF lot-8 & 9 is included in the process. - Two new impurities, Monofluoro CTPM2 and Difluoro CTPM2, are included in IPC no. CTPM2-09 and intermediate analysis. - In section no. 3.5a and 3.5b, IPC results of new lab assurance batches and intermediate analysis are updated. - In Section No. 2.3, the impurity table is updated with new impurities along with revised RRT values. - In Section 2.4, the impurity profile from new lab assurance batches is included. |
| RP-CDM-000737-8.0 (CCF No.: CC1000117293) | The changes below are considered under the current document. - CTPM2 reaction procedure Section 3.1, Op. No: 104, 113 and Section 3.5 a) IPC CTPM2-09 Purity by HPLC: NLT 98.0 %, (Limit: Related substances by HPLC (% w/w), Method-I: i) DMAP: NMT 0.4 ii) Monofluoro analogue of CTPM2: NMT 0.4 iii) Difluoro analogue of CTPM2: NMT 0.4 iv) Compound 7: NMT 0.6 v) Boc-L-valine: NMT: 0.4 vi) CTPM1: NMT 0.4 vii) Impurity at about 0.93 RRT: NMT 0.5 viii) Individual unspecified impurities: NMT 0.4; Method-II: i) Dicyclohexylurea: NMT 0.5 - Section 3.5 b) Complete analysis of CTPM2 intermediate Appearance: White to off-white color, solid Identification by FT-IR was removed from the intermediate specification. Related substances by HPLC (% w/w) Purity: NLT 98.0 %, Method-I: i) DMAP: NMT 0.4 ii) Monofluoro analogue of CTPM2: NMT 0.4 iii) Difluoro analogue of CTPM2: NMT 0.4 iv) Compound 7: NMT 0.6 v) Boc-L-valine: NMT: 0.4 vi) CTPM1: NMT 0.4 vii) Impurity at about 0.93 RRT: NMT 0.6 viii) Individual unspecified impurities: NMT 0.4; Method-II: i) Dicyclohexylurea: NMT 0.5 |
| RP-CDM-000737-9.0 (CCF No.: CC1000126425) | 1. The 0.93RRT impurity structure name was updated in the impurity profile of the CTPM2 table under section 2.3. 2. The impurity at about 0.93 RRT was updated as “0.93 RRT impurity of CTPM2” in IPC(CTPM2-09) and intermediate analysis in line with spec and MOA. |
| RP-CDM-000737-10.0 (CCF No.: CC1000XXXX) | - As per recent FMEA-RA, NOR, and PAR studies, the critical process parameters (CPPs) have been reclassified as note points. - Incorporated newly conducted what-if experiments and updated the stoichiometry of reagents experiments in the optimization section. |